NOTE:  Life rafts/dinghies are a last resort and should be deployed only when you run out of other options.

NOTE:  The "mother ship" is always easier for rescuers to see.

When to Abandon Ship?

Only abandon ship if the vessel is:

- actually sinking

- has an uncontrollable fire

- someone's life is endangered by staying aboard

Preparation

- Don life jackets

- Write down present position.

_____o _____. ________’ N

_____o _____. ________’ W

- Send out MAYDAY message. (See MAYDAY tab)

- Activate EPIRB

- If time permits:

  - drink as much fluid as possible

  - use the head

  - put on warm clothes

- Gather the following items –Abandon Ship Bag/Survival Kit (under seat on stbd side of flybridge)

Portable VHF

Manual inflation pump

Flashlight

Binoculars

Bucket

Cell phones

Flares

Warm Clothes, gloves

Extra rope

Prescription meds

First Aid Kit

Multitool

Duct tape

Sunscreen

Bug repellent

Ziploc bags (gallon size)

Emergency procedures

Extra batteries

Paper & pen in Ziploc bag

Cushions from salon, staterooms

Extra water,

Extra food

Oars for dinghy,

personal items:

money,

passports,

glasses

Hats, gloves

Boat documents

********************************************************************************

IF Captain has made decision to abandon ship, THEN take the following ACTIONS:

- Launch dinghy -  make sure dinghy is tied to “mother ship” when it is launched

- Get into dinghy

-  try to enter from boat

-  first person into dinghy should be the most physically capable person so they can pull others into dinghy from water if necessary.

-  cut loose from “mother ship” at last possible minute

- Stay as close as possible to site of sinking. (per GPS)

- Collect all available flotsam.  Tie large floating debris together to make it easier to spot from the air.  The most unlikely articles can be adapted for use under survival conditions.

- Keep warm and dry, especially your feet.

- Stream a sea anchor.

- Setup lookout watches.

If Dinghy Not Available

- Stay with boat, if at all possible - easier to see from air.

- Lash several silver "space blankets" to hull to improve visibility of black hull from air.

- Find loose lines and secure them to cleats under hull, then throw line over hull to other side, secure to opposite cleat. Do this in several places along the hull to serve as handholds for climbing and hanging onto hull.

Use flares only on skipper's orders - when there is a real chance of them being seen.

Signaling

- Use signal mirror, whistle, strobe-light if available.

- Flares

- IF other boats are in area or aircraft is flying over, THEN shoot two flares off back-to-back.

- Point parachute flares upwind of boat at 15 degrees off vertical and shoot.

- Hold flares horizontally using a glove outside dinghy – the hot slag drops from flare.

Water Rationing

- Collect rainwater

- Ration water to maximum one-half quart/person/day, issued in small increments.

- Do not drink seawater or urine.

- If water is in short supply, then eat only sweets from survival rations.