DEFINITIONS

- Mayday - Distress call for grave and imminent danger to life or vessel

- Pan-Pan – (pronounced "pahn-pahn") Declare an urgent situation, but does not pose an immediate danger to life or vessel

- Securite – (pronounced say-curi-tay) announce a navigational safety message

WHEN TO USE:

- Mayday – The boat is sinking, fire in the engine room, or someone is unconscious or experiencing a serious injury or illness

- Pan-Pan – The boat has a slow leak; the engines are disabled, and the boat is drifting to rocky shore

- Securite – Announce a submerged log in channel, entering a narrow channel, or navigational aid missing

TO TRANSMIT CALL:

  - Turn on VHF Radio

  - Select Channel 16

  - Verify VHF Antenna is raised.

  - Press and hold the TRANSMIT button on mic.

  - Hold mic on side only (don't cover up holes on front) approx. 2" from lips.

  - Speak slowly, calmly and clearly the message script.

  - Release "TRANSMIT" button on mic.

SCRIPTS

- Mayday

  - MAYDAY - MAYDAY - MAYDAY

This is motor vessel Still Waters II

Our position is (Look for "GPS Position" on Chart plotter display (on flybridge))

North Latitude __ degrees, __ minutes, __ seconds

West Longitude __ degrees, __ minutes, __ seconds

We (describe nature of emergency)

We require (describe assistance needed)

Onboard are (describe # and type of persons onboard -" 4 adults, 2 children", etc)

Still Waters II is a 43-foot Aft Cabin Motor Yacht

I will be monitoring channel 16

MAYDAY-MAYDAY-MAYDAY

This is Still Waters II, over

- Pan-Pan

  - PAN-PAN    PAN-PAN   PAN-PAN

All Stations, this is motor vessel Still Waters II

Our position is (Look for "GPS Position" on Chart plotter display (on flybridge))

North Latitude __ degrees, __ minutes, __ seconds

West Longitude __ degrees, __ minutes, __ seconds

We (describe nature of emergency)

We require (describe assistance needed)

Onboard are (describe # and type of persons onboard -" 4 adults, 2 children", etc)

Still Waters II is a 43-foot Aft Cabin Motor Yacht

I will be monitoring channel 16

PAN-PAN    PAN-PAN   PAN-PAN

This is Still Waters II, over

- Securite

  - Securite Securite Securite

All stations, this is motor vessel Still Waters II

(State message)

Still Waters II – out

  - Example messages

  - Entering a rough inlet:  ….. entering Jones Inlet. I am restricted in my ability to maneuver. Please stand clear.

  - Serious Mechanical Problems: ..….traveling (or dead in the water, without propulsion, etc.)(direction) My position is XXXXXX latitude, XXXXXX longitude at/off/near (location). We have (describe problem) and are attempting to fix it ourselves (or request assistance from TowBoatUS/USCG).

  - In Restricted Visibility/ Limited Maneuvering: …..traveling (direction) on course (XXX) degrees, in (zero, limited) visibility at X knots (mph) at/off/near (location). My position is XXXXXX latitude, XXXXXX longitude. My destination is _____.  Standing by for any concerned vessels on channel 16.

  - For Discourteous Boaters:  …..travelling (direction, northbound, etc.) on at/off/near (location). This is a warning for all vessels traveling (Location). A (description, make, model, color, type) vessel by the name of “GoFast” is travelling (direction) on (location) at a high rate of speed dangerously close to other vessels causing potential harm to vessels and crews in the area. All vessels in (this area) should be on the lookout for the vessel “GoFast”.

TO CANCEL CALL:

If you find you are able to handle the emergency without assistance and have not delegated control of the situation to another vessel or station, you must cancel the MAYDAY call whether or not it was acknowledged.

- If it was acknowledged by USCG or another authority, contact them in a non-emergency way and inform them of the cancellation.

- If it was not acknowledged, cancel Mayday using the format similar to Pan-Pan cancellation

PAN-PAN, PAN-PAN, PAN-PAN

All stations, all stations, all stations

This is motor vessel Still Waters II

The time is _________

Cancel Pan-Pan

This is Still Waters II, out