Professional estimates are that you have 30 - 60 seconds to start fighting fire before it gets out of control on a boat.

Systems

Smoke Alarm

Still Waters II is equipped with a smoke alarm located in the dining area.

Fire Detection and Alarm

Still Waters II is equipped with a fire detector located above the aft end of the port and starboard engines.  The alarm sounds at the helm.

Fire Suppression Equipment

Halon Suppression System is in the engine room, bottle is on starboard side in front of the starboard engine.

Manual Pull Station is in the helm, port side, next to VHF Base Station.

Fire Extinguisher Locations

- V-Berth, starboard, hanging closet

- Galley, starboard, outboard of refrigerator

- Captain Quarters, starboard, hanging closet, forward of electric cord reel

- Helm, starboard, forward corner

Class of Fires

|  | Class | Type Fire | Extinguishing Agent |
| --- | --- | --- | --- |
| P | A | Paper, leaves ash | Water |
| O | B | Oil | Dry Powder |
| E | C | Energized electrical | Halon |
| M | D | Metal | Special Agent |

  - White smoke - usually indicates electrical/paper/cloth fire

  - Black smoke - indicates fuel fire or fiberglass

  - Do not use water to extinguish electrical or grease fire - Use ABC extinguisher.

How to Use Fire Extinguisher

Use PASS method.

Pull Pin

Aim at base of fire

Squeeze handle

Sweep side to side

PRECAUTIONS

  - Do not open engine room hatch if flare-up is possible.

  - Use cloth saturated with water over mouth or cloth filter masks to breathe smoke filled air.

ACTIONS

Report Fire

  - Yell - "FIRE (or WHITE/BLUE/BLACK SMOKE) in ________." Repeat, 2x.

  - Place a "MAYDAY" call on VHF radio (see MAYDAY procedure).

  - Don LIFE JACKETs and shoes.

  - Gather available portable extinguishers and take to vicinity of fire.

For Engine Room Fire –

  - Pull the safety pin to activate the Automatic Engine room HALON System

  - Check light to verify system has activated.

- This should automatically shutdown both engines

- Will NOT automatically shutdown GenSet if it is running.

  - Shut it down with switch in Electric Panel.

    - Shut down:

    - engines

    - generator

    - Air cond. Systems

    - Open AC panel breakers

    - Open DC panel breakers

    - Shut down inverter

    - Turn off AC power to boat at dock power pedestal

NOTE:  IF fire appears to be out, THEN wait at least 15 min. before opening ER hatch

    - Engines can be restarted in emergency by putting Fireboy system switch (on lower helm dash) in “Override” position.

    - If fire still burning - use portable fire extinguisher next, but be very careful opening ER hatch.

- Feel hatch/door for temperature before opening. (Use back of hand)

- If cool or just warm open door just enough to put in nozzle.

- If too warm or hot consider discharging extinguisher into Engine air suction vents on port & stbd side of hull (black louvers).

- Crouch low with door between you and fire.

FOR ELECTRICAL PANEL FIRE (salon)

    - Turn off AC power to boat at dock pedestal

    - Turn off battery charger on Inverter Panel on port side of salon

    - Turn off Solar Panel charger to batteries on port side of salon

    - Shutdown GenSet using switch on Main Electric Panel on starboard side of Salon

    - Turn off battery isolation switches in ER on aft bulkhead above house batteries.

FOR ANY OTHER FIRE

- Grab nearest ABC extinguisher & start fighting fire

- Continue using additional extinguishers until fire is out (4 total onboard)

- If possible, head the boat so that the flames blow outboard rather than inboard.

- Start preparing dinghy for launch. (See Abandon Ship Tab)

- Move crew to safest part of boat (bow, if turned into wind).

AFTER FIRE IS OUT

  - Thoroughly ventilate compartment(s) of smoke and extinguishing agent. Wait at least 10-15 min. before entering compartment.

  - Carry an unused fire extinguisher and don protective clothing, cloth face mask and gloves.

  - Enter compartment, assess damage, set re-flash watch.

  - Notify emergency responders if they are no longer needed.

  - Override fuel shutoff solenoid to restart undamaged engines if needed.