Flooding Systems

Bilge Pumps

Four bilge pumps on board.

| Location | Type | GPH |
| --- | --- | --- |
| Bow – under hatch outside fwd shower | Float switch |  |
| ER – between ME’s | Manual switch 1 |  |
| Captain’s Quarters – in fwd hanging locker | Automatic 2 |  |
| Captain’s Quarters – under bed, stbd, along transom | Float switch |  |

1 – Can be started from switch on helm or from breaker in Electric Panel

2 – Pump starts automatically every 2.5 minutes and continues to run as long as there is discharge pressure

Two Shower Pumps on board can be converted to bilge pumps.

| Location | Type | GPH |
| --- | --- | --- |
| Galley – under dinette seating along fwd bulkhead | Float switch 1 |  |
| Captain’s Quarters – in fwd hanging locker | Float switch 2 |  |

1 – Turn breaker on in Electric Panel (Salon) and switch in fwd head (above sink)

2 – Turn breaker on in Electric Panel (Salon) and switch in aft head (aft of sink)

ME’s can be converted to bilge pumps using the ‘crash valves.’

General Information

- 1 gallon of water weighs 8 pounds

- 1 cu ft of water weighs 62.4 pounds

- Still Waters II weighs 34,000 pounds dry with 3 ft 9 in draft

- It takes approx. 2210 lbs of water to lower waterline of Still Waters II 1” in fresh water.

- Or it takes about 275 gal. of fresh water to lower waterline 1”.

- With all bilge pumps running full blast, with a loss of a single shaft (2.25” hole), net amount of water accumulating in boat will be about 6000 gph. At that rate Still Waters II will sink nearly 2’ at the waterline in that hour and will most likely lose all its reserve buoyancy.

Flooding Rates of Various Size Holes

Figures Are in Gallons Per Minute

| Hole Diameter | 6" Below waterline | 1' Below Waterline | 1' 6" Below Waterline | 2' Below Waterline | 3' Below Waterline |
| --- | --- | --- | --- | --- | --- |
| 1/8" | .17 | .30 | .31 | .35 | .43 |
| 1/4" | .88 | 1.20 | 1.53 | 1.80 | 2.20 |
| 3/8" | 1.94 | 2.70 | 3.40 | 3.90 | 4.80 |
| 1/2" | 3.46 | 4.90 | 6.00 | 6.90 | 18.50 |
| 3/4" | 7.77 | 11.00 | 13.50 | 15.60 | 19.10 |
| 1" | 13.96 | 19.60 | 24.20 | 27.80 | 34.0 |
| 2" | 55.49 | 78.60 | 96.10 | 111.10 | 136.10 |
| 4" | 222.10 | 314.30 | 378.70 | 444.50 | 544.40 |
| 6" | 499.60 | 707.20 | 865.30 | 1000.20 | 1225.00 |

Pasted from <http://www.marinesurveyorschool.org/pages/hullpenetrations.html>

Ideas to Stop Flooding

    - Recycle the wreckage - try using the broken piece from the hole to plug the hole.

    - Drive wooden conical plugs wrapped in a rag into hole.

    - Use a brace to hold broken piece of fiberglass or use it to push a piece of wood (ER seat, shelf, dinghy oars) against hole. Use cushion, toilet wax, foam, etc. under plywood for better seal.

    - Place a hole in center of a piece of plywood, feed through a rope with knot on the end, place over hole in boat from outside the hull and feed thru rope to inside of hull and tie off rope inside to tighten board on outside of hull.

    - For a long hole or crack in hull use sofa cushion or foam rubber (under mattress).

    - Thread one or more poly lines with small floats on the end, out the hole from inside the boat, take the floats off the lines and pass them through the holes in 2 ft square 1/4 plywood, then, from inside the hull, pull them back tight to the hull.

  - Rolled up towels fill a small hole well, also pillows and cushions.

  - Place cooking pot with caulking over object protruding through hole and shore in place.

  - For small crack in hull drill hole in each end to stop propagation.

  - Use water pressure to block leak from outside the hull - rubber backed welcome mat, vinyl covers, cushions, big trash bags, etc.

  - Move water or fuel to list boat towards side away from hole.

  - Bicycle inner tubes can be used to cover hose splits

  - Split a soda can along its length, remove top and bottom of can to wrap around a split hose with inner tube underneath can. Secure with hose clamps.

  - When you need to replace a ruptured hose, but the only spare hose onboard is too large in diameter, cut a temporary replacement from the larger hose, then cut two short lengths. Split these short sections longitudinally and remove enough material from each, forming them to the required smaller diameter. Insert these "bushings" into each end of the temporary replacement, install the hose fittings and replace hose clamps.

  - Soak a rag in epoxy putty and jam it into the hole.

  - Jam something rubber (a ball, foam, cushioning, fender) into the hole.

  - Stuff a greased rag or towel or carpeting into the hole, and tap a peg into that.

  - From inside, put cushion over hole, flat board over cushion, jam boat-hook against board.

  - From outside, stick a "plumbers thumb" or rubber plug or cork into the hole, or stick a toilet plunger head over the hole.

Convert ME to Bilge Pump

    - Just use one engine at a time.

    - Establish communication with person at helm

    - If navigation situation is OK, shutdown engine that will be used.

    - Close engine raw water seacock.

    - Open engine ‘crash valve’

    - Start engine and observe that it is getting adequate water flow.

      - If necessary, crack open seacock to supply additional water.

      - Watch temp. gage.

      - Turn off engine if temp. rises above 210

Action

LOOK, SMELL, LISTEN BEFORE ENTERING FLOODING COMPARTMENT.

  - Put on LIFEJACKETS and SHOES.

  - If in grave and imminent danger of sinking - BROADCAST "MAYDAY" on CH. 16 - See MAYDAY procedure. Otherwise notify Coast Guard by radio that you have a situation onboard and to setup a "radio watch" to check in, every 15 minutes.

  - Open RED cover and press DISTRESS button for 5 seconds on VHF radio on flybridge panel. The radio will automatically listen on Channels 16 and 70 for a response. (see MAYDAY procedure for more details)

  - Assign someone to gather Ditch Bag and prepare dinghy for launching. (see Abandon Ship proc.)

  - If Severe Flooding - consider GROUNDING BOAT IN SHALLOW WATER to keep from sinking.

  - Assign someone to work VHF Radio - Locate portable VHF radio and take to flybridge.

  - Start Engines - If water hasn't reached engine room

  - Check that bilge pumps are working on Alarm Panel. If not running – Start pumps w/switches at helm, Electric Panel, or locally.

  - Lots of water in bilge? Shutdown GenSet and Inverter before looking for leak to avoid electrocution.

    - Is deck slippery? Smell chlorine gas? What's running in ER?

  - Once in compartment, find leak and attempt to plug, check operation of bilge pumps.

  - Shut seacock to broken hose if applicable.

  - If unable to control flooding - send out "MAYDAY" & start "ABANDON SHIP" procedure.