PREPARATION

  - "Restricted Visibility" = fog, heavy rain, snow, sandstorm, or smoke.

  - Navigation Rules state to take actions "in or near" area of restricted visibility.

  - Fix your position before entering fog - GPS, visually, bearings, etc.

  - Verify proximity to hazards.

FOG SIGNALS

Range of typical ship's fog signal at sea is about 2 miles

- Definitions

  - "Short blast" = 1 sec.

  - "Prolonged Blast" = 4 - 6 sec.

  - “Required Interval” = two minutes or less between signals. (Shorter intervals are desired when other vessels are near)

- Vessel Underway -  1 prolonged blast every 2 min. or less

- Vessel "underway but stopped in water" – Prolong Blast -2 sec. wait - Prolong Blast - do this every 2 min. or less

- Anchored vessel - ring bell rapidly for 5 sec. - do this every 1 min. or less. Boat may additionally sound a 3-blast whistle signal - one short, one prolonged, one short to give warning of its position and the possibility of collision

- Vessel aground in fog - 3 separate & distinct strokes on bell -ring bell rapidly for 5 sec.-3 separate strokes on bell - do this every 1 min. or less. May also sound appropriate whistle signal upon detecting approach of another vessel.  Morse code for letter "U", short-short-long.

Other Fog Signals:

  - Towing vessel - 1 long, 2 short every 2 min. or less.

  - Vessel being towed -  1 long, 3 short, every 2 min. or less. (ideally should be right after towing vessel)

  - Pilot vessel (when engaged in pilot duties) - 4 short blasts (in addition to normal underway signal)

  - Vessels under 39' need not make specified sound signals, but must make "some other efficient sound signal every 2 min.

Lookouts

    - Bow lookout - watch for other vessels, nav. hazards, listen for engine noise, sound signals.

    - Stern lookout - watch for overtaking vessels, listen for fog signals.

    - Lookouts should use all their senses:

      - Listen! - Very thick fog on windless days can bounce sounds. A foghorn may appear to shift in direction. Be sure to carefully verify direction over 10 minutes or so of close listening.

      - Look! Watch for unusual waves which might indicate a passing boat close aboard.

      - Smell! Be sensitive to smells - land, other boat's exhaust.

Actions

  - Don life jackets and shoes, If in heavy traffic area

    - Station Lookouts

  - Start a "go to" route by placing a target to the cursor.  Note compass heading.

  - Turn on navigation lights.

  - Check spotlight and bring portable spotlight to helm.

  - Turn on radar:

    - Set scale commensurate with visibility

    - Set proximity alarm.

    - Start appropriate fog sound signals for your vessel’s status when you can’t see as far as your sound signal can be heard. (about 2 miles)

  - Reduce speed to minimum to keep on course.

  - Occasionally, when safe to do, stop engines and have everyone listen intently for other ship noises.

  - Monitor AIS & VHF Ch. 16 & 13.

    - Occasionally broadcast "Securite’" calls with ship's type, name, course, position and destination on Ch. 16 & 13 - use 2 separate radios to monitor both channels.

    - Example: "This M/V Still Waters II traveling at 3 knots on a course of 080, located approx. 1/2 mile east of lighted Green 57 at the entrance of XYX River enroute to Bob's Marina. This is Still Waters II standing by on Ch. 13 & 16. Out."

  - If called on VHF, respond with something like "This is M/V Still Waters II, I'm the target approx. 1/4 mile east of Red 14 in the Indian River, proceeding at 3 knots on a course of 180 degrees, and I do (or don't) see you on my radar." (Who I am, where I am, my course & speed, do I see you?)

  - Consider anchoring - but not in or just outside channel.

      - Best anchor location is with a bunch of other boats well out of channel.

      - Display anchor light - turn off nav. lights.

      - Be prepared to retrieve or cut anchor quickly.

  - Reduce speed to bare steerageway or stop if fog signal is heard forward of beam

    - Do not alter course until position of other vessel is known with reasonable certainty.

OTHER POSSIBLE ACTIONS TO TAKE:

  - If with a group of boats - travel single file with each boat towing something behind it (milk bottle, empty oil bottle, etc.). Each boat follows other's towed object.

  - Consider staying away from buoys that typically draw GPS equipped boats towards them in fog.

  - On Great Lakes - To activate radio-controlled fog signals on certain AToN's (Aids To Navigation), tune to VHF channel 83A and key the VHF radio mike 5 times.