CONSIDERATIONS: TOWING vs SALVAGE

TOWING - soft groundings, no immediate peril to ship or passengers, water pump out not req'd.

SALVAGE - Ungrounding that involves Risk or Peril, peril to vessel and or passengers.

See SALVAGE page

To avoid grounding:

    - Determine course from dock anchorage back to main channel.

    - Keep close eye behind boat - take back bearings on previous buoy to make sure you are not out of channel.

    - Do NOT trust the magenta line on chart plotter, Honor the Buoy if there is a discrepancy

    - When following another boat, watch the boat's wake as it approaches shore. Watch where the wake "breaks" – changes from a smooth wavelet to miniature white topped breaker - that's the point where water rapidly shoals.

    - When following another boat, follow the channel, not the stern of the boat

    - Watch the water for signs of shallowing water: ripples, difference between running and standing water, white foam line along water.

    - Deep blue water changing to various shades of green - lighter color can indicate areas of shallower depth.

ACTION

  - Immediately put throttles in Neutral.

  - Shut down engines (don't want to suck up sand)

  - Have all passengers don life jackets, shoes and gloves.

  - Check for crew injuries. If serious injuries have occurred, call for assistance.

  - Inspect engine room and other bilges for leaking shafts, rudders, through hulls, etc.

  - If leak is detected - see FLOODING - probably should NOT try to refloat. Call for HELP!

  - If leak is not initially detected, periodically check again for hours after grounding.

  - Shut down AC and GenSet if you are sucking up sand. check strainers.

  - Record location, time and note wind and tide conditions.

  - Probe with boat hook to determine depth around boat -where is deep water?  What kind of bottom is it? Mud? Sand? Look at normal waterline mark around boat for clues about severity of grounding.

  - Put up Marine Distress Flag (in seat locker by flybridge sink- red with black square & black circle)

  - Consider calling TowBoatUS on Channel 16 VHF or call local TowBoatUS office on phone

    - BOATUS Member # XXXXXXXX Expires;  XX/XX/XX  National Ph. # 800-391-4869

    - Unlimited freshwater towing - must be within a towing service area to cover 100% of cost to tow from sea to home dock or repair facility; up to $2,500 cost covered outside towing areas when authorized by BOATUS. (see Agreement in boat documentation).

    - Our Yacht Insurance covers Towing up to $5,000.

    - If not TowBoatUS, pass towline to other boat to avoid potential disputes over salvage rights.

  - Consider calling USCG for help if we are a "hazard to navigation".

  - If in high traffic area - place a Securite' call every 15-30 minutes advising that you are aground and give location.

  - Tide - If Tide is Rising - wait (tide rises and falls in approx 6 hr cycles)

  - Tide - If tide is falling,  make sure boat will tilt to uphill side by using anchors, dinghy hoist, etc.

    - Duck tape all vents, holes, hatches, windows, door, openings, lockers etc.

    - Turn off fuel, power and propane.

    - Protect hull from rocks using cushions.

    - Turn off bilge pump float switches if heeled over sharply and pump is running without water.

    - If thinking of calling a towing service, carefully consider when they should be called since they may just have to wait until tide rises to pull you off.

  - Stay away from tow ropes under strain. Consider using rope bridle to two bow or stern cleats to spread load. Double braided nylon rope is preferred for towing.

  - Call Insurance Agent if damage is suspected (Insur. Co. # _____________; after hrs. _______________)

  - Take pictures.

  - Sound Signals - reduced visibility and aground - Ring bell (one stroke -ring rapidly for 5 sec - one stroke) every minute or sound short blast - prolonged blast-short blast on horn. Fly anchor ball in daytime, anchor light on at night.

  - If you must abandon boat:

    - Leave boat using shutdown checklist.

    - Close all seacocks.

    - Double check bilge pump settings. Turn off bilge pump float switches if heeled over sharply.

    - Try to do underwater survey for damage: rudder, props, hull, best route.

TECHNIQUES

  - Lighten boat - pump out FW tanks; shift heavy objects (including crew) to deep water side or put in dinghy and take to shore.

  - Rock the boat - If engines can be safely started (watch temp gages & make sure you’re not sucking up sand or mud into raw water, check strainers), start with engines in reverse and twist wheel back & forth. If not working - STOP. Try going forward if not shallower and do same thing. Go Forward - Reverse - Forward - Reverse to rock boat. Use bow thruster to turn bow.

  - Boat Wakes - Try timing engine use with passing boat wakes. Call them – request they make wake.

  - Lighten boat by removing water and fuel (800gal. fuel x 6 lbs/gal + 200 gal. water x 8.3 lbs/gal) =6,400 lbs. (do not offload fuel to river/ocean etc.)

  - Scouring using high-pressure hose, props in forward (if anchored).

  - Take a Kedge Anchor to deep water - to keep boat from going more aground - take out in dinghy.

  - Try, at least, to get bow pointed to deeper water. If wind or tide is pushing boat towards deeper water try to put stern into wind or tide to help generate more force on boat.

  - Cushion boat sides - if tide is going out and there is danger of boat heeling over on a chine - use boathooks, oars, cushions, etc.