Definition – Conditions with winds 28-33 knots (32-38 mph) or more and a wave height of 12 feet or more

Judgement of Navigation in Heavy Weather

- Obtain forecast for the area, continue to monitor weather

- Develop Passage Plan

- HEAD FOR SAFE HARBOR, cove, or sheltered (lee) side of shore or island.

- Is there time?

- Consider if going back to the dock can lead to other problems like missing (or hitting) channel markers, running aground, hitting the breakwater, not being able to dock due to high winds. Can we dock at fuel dock?

- Look for good bottom holding shoreline.  If safe to anchor, set anchor properly, and use engines to motor forward just enough to reduce strain on anchor rode during periods of peak wind velocity.

- HOLD STATION

Prepare

  - Don Life Jackets & Shoes.

  - Brief the crew to explain the situation and reassure them.

    - Make sure that they know what to do, and what not to do, when the heavy weather arrives.

    - Explain such things as keeping low in the boat, not moving around excessively and not going out on deck unless necessary.

    - Give them all an assignment to keep them occupied and keep their minds off the situation.

  - Turn on Navigation Lights.

  - Secure all loose gear above decks and below.

  - Put away small items and lash down larger ones.

  - Latch refrigerator, freezer doors.

  - Close windows.

  - Remove & Secure loose items from sun deck.

  - Tighten up dinghy tie downs

  - Distribute motion sickness pills - more effective if taken before you need them.

  - Don appropriate foul weather gear.

  - Make anchor ready to let go

  - Monitor local NOAA weather radio, weather radar for possible paths around storm.

  - Set separate Look Out to watch for other boats and floating debris.

IF LIGHTNING IS PRESENT

- If < 30 seconds between seeing lightning and hearing thunder, boat could be struck  (5 sec = 1 mile away)

- Lower VHF antennae

- Move everyone below, stay low in boat, and stay away from metal objects

  - Start back-up navigation app on iPad if not already running

- Wear rubber gloves when using wheel.

- Put portable VHF, phones, and iPads inside microwave (Faraday Cage)

- If in familiar waters, consider unplugging all wires from back of Chartplotter

HEAVY WEATHER MANEUVERING

Meeting Head Seas

- Slow speed in order to ride up and over the waves, rather than driving the bow into the waves

- IF wave conditions cause the bow to bury in the wave, THEN slow down to make bare steerage way and hold your bow 45 degrees to the swells

Running in the Trough (parallel to the waves)

- Make a series of tacks taking the waves at 45 degrees on the bow and then 45 degrees on the quarter

- Minimize the time in the trough when making the tack

Running Before the Seas

- Keep the stern perpendicular to 15-degree angle to the swells

- Always try to ride the back of a swell, never the front or face.

Other Considerations

- Don't steer a straight course. Steer a path of least resistance through waves.

- To make progress in direction you want to go, tack through waves at 30 - 45 degrees off bow and then off the quarter.

- Avoid highest crests as much as possible.

- Check nearby water depth, navigation hazards, ship traffic.

- Broadcast PAN-PAN, PAN-PAN message on Ch 16 & 13 giving location if near other vessels or shipping channels (see PAN-PAN procedure).

- When steering, don't overcorrect. Make small changes with helm or throttle.

- If you have to turn, do so with care. It's best to avoid major turns if you can. But if you must turn, edge over to an angle of about 15 to 25 degrees off the crests and watch for an opportunity. Wait for a lull, just after a crest passes your boat. Turn as quickly as possible using rudder and engines.

- A typical cruising style motor yacht may be considered at risk, if in a fully developed beam sea, the combined wind wave and swell height of the sea is equal to or greater than 20% of the water line length of the vessel. (8' - 10' seas for Still Waters II)