GENERAL: Helo rescue typically is only possible within 100 - 300 miles of shore facility depending on type of equipment available.

- Don life jackets.

- Lash & Stow all loose gear from bow to stern

- Tie down or bring in fenders

- Remove flagstaff.

- Lower VHF antennas.

- Establish communications with helo on portable VHF radio as directed.

- Remove hats of all types (can be sucked into helo intake)

- Move all hands not needed for the operation should be inside boat, below decks, far forward.

- Steer into the wind, glance at compass heading and add 30 degrees (puts wind 30 degrees off port bow). Steer that new course if seas allow. (Helo pilot makes final call for direction to steer). Maintain a moderate speed.

- Let the Trail Line (steel cable hanging down from basket/stretcher/pump) hit the deck first to discharge static electricity.

- If necessary, disconnect the equipment from the line as quickly as possible. Do not attempt to move the rescue device while attached to the cable.

- DO NOT attach line to anything permanently mounted to boat.

- Signal helo that line is disconnected and can be raised.

- Place person (with PFD) in rescue device sitting with hands clear of sides or strapped in, face up.

- Reconnect hoist line when rescue device is loaded.

- Signal hoist operator in helo with "thumbs up" when ready.

- Once hoist begins, maintain boat's course and speed.

Medical Evacuation

- Coast Guard can arrange for free medical advice if warranted.

- Final decision for a Medical Evacuation (MEDEVAC) rests with the Coast Guard Operations Center and evaluation of symptoms by a USCG Flight Surgeon.

- Be prepared to give the following information to the USCG:

  - Vessel's name, call sign and position

  - Course, speed, next port of call, and ETA

  - Patient's name, nationality, age and sex.

  - Patient's respiration, pulse and temp.

  - Symptoms and nature of illness

  - Location and type of pain

  - Medical supplies carried aboard

  - On scene weather

  - Communications schedule, frequency and capabilities.