DEFINITIONS:

- Ebb Tide: Tide is flowing out of the inlet.

- Flood Tide: Tide is flowing into the inlet.

- Slack Water: The time between the ebb and flood tides when the current is relatively slow.

- Diurnal ("Daily Tide"): One high and one low tide per day.

- Semidiurnal: Two high and Two low tides per day - 2 complete cycles.

- High Slack Water: The slack water that occurs when the tide is changing from flood to ebb and the water level is relatively high. Occurs at the end of the flood tide.

- Low Slack Water: The slack water that occurs when the tide is changing from ebb to flood and the water level is relatively low. Occurs at the end of the ebb tide.

- Inlet “Bar”: Dangerous sandbars at the mouth of inlets and bays.

CONSIDERATIONS:

Timing the Tides

    - Ebb tide plus outgoing river current = very strong current outflow + larger, steeper & closer together waves. This condition can subside dramatically when the tide turns.

    - Ebb tide + strong onshore winds = bad conditions.

    - Be aware of offshore storms that might create large waves at inlets. Offshore storms might be totally gone but their effect might still be showing up at the inlet.

    - Check tide tables (electronic & paper) to select right time to cross.

      - Know what corrections must be applied to the location of the tide station to find the tides at the bar you are going to cross.

      - Are the tide tables adjusted for daylight savings time?

    - Try not to cross during ebb tides.

    - Try to avoid crossing unfamiliar bar at night or in fog/low visibility.

    - Generally, safest time is the relatively short period of slack water between ebb and flood tides. Diurnal (daily) tides = 2 chances to cross. Semidiurnal = 4 chances to cross.

      - With semidiurnal tides try to choose the tide with the least difference between high and low tides.

      - Compare tidal variation on day you plan to cross to typical tidal variation. Generally, the more variation, the more likelihood for rough conditions.

    - Transiting at Low Slack Water

      - Advantages:

        - Works best for longer bars.

        - Entering from ocean side gives you more time to cross (more push on the flood).

        - Rising tide might help free boat if you run aground.

    - Transiting at High Slack Water

      - Advantages:

        - Water on bar is the deepest at the end of the high tide. Deeper water means a wider safe channel - reduces the size and steepness of the waves.  Current should be minimal.

Check Bar Conditions:

- Check river bar runoff conditions. High runoff (floods, rains, snowmelt) = more severe conditions.

- Deep, narrow inlets are more likely to have a fast and strong current.

- Check sea state outside of inlet. Large swells or wind waves = more severe conditions.

- Listen to local NOAA Weather Radio reports for inlet conditions. (like time of max. ebb current - AVOID!) www.nws.noaa.gov/nwr/coverage/station_listing.html

- Check Local Notice Mariners for last 30 days and the East Coast Alerts. (Can sign up for East Coast Alerts (via email) at:  https://www.boatus.com/cruising/tomneale/signup.asp)

- Examine charts - look for shallow spots surrounded by deeper water - good places for big breakers to exist.

- Be wary of entrances with channel water depth < 30'.

- Observe entrances (bars) for a while before making the crossing. Radar will often show breakers, if they are in the channel or nearby, even in the dark. On the radar they look similar to lines of small seagull symbols, moving towards the beach.

- If unsure of current conditions or your skills, consider hiring a local captain to cross.

- Notice wave pattern through inlet - large wave followed by series of smaller wave.

- Some problem inlets:   Fort Pierce, FL - strong outflow current

Where to Cross:

- Always start approach from sea buoy - best place to scope out conditions in inlet.

- In general, it's unwise to attempt to cross unmaintained or unmarked bars without expert local knowledge and experience.

- Electronic and paper charts often show the sea buoy offshore, but will not show any aids to navigation in unstable inlets, even though they are there. This is because they must be frequently moved to keep up with shoaling changes.

- Keep a very alert watch - stay clear of large commercial vessels. Hail on Channel 13.

Preparing for Crossing & Cross:

  - Don life jackets

  - Watch for dredges. Will display 2 green lights or 2 diamond shapes in vertical line to indicate side that is safe to pass. Floating pipes are usually marked with orange or yellow lights.

  - Prepare anchor for immediate deployment, remove rain cover

  - Be prepared to stay outside inlet for a tide cycle or cycles.

  - Get experience in handling boat in large following or head-on seas while outside inlet.

  - When entering inlet, adjust speed to stay on back of a breaking wave.

  - If you must turn around after into inlet, start turn when largest wave passes under boat and complete before next large wave hits.

When in doubt, Stay Out!